SFP-1000BASE-T Copper SFP Transceiver

1.PRODUCT FEATURES

- Up to 1.25 Gb/s bi-directional data links

- Hot-pluggable SFP footprint

- Low power dissipation(1.05W typical)

- Compact RJ-45 connector assembly

- Fully metal enclosure, for lower EMI

- RoHS compliant and lead-free

- Single +3.3V power supply

- 1000BASE-T operation in host systems with SERDES interface

- 1.25 Gigabit Ethernet over Cat 5 cable

- Ambient Operating temperature: 0°C to +70°C

2.PRODUCT DESCRIPTION

SFP-1000BASE-T  Copper Small Form Pluggable (SFP) transceivers are based on the SFP Multi Source Agreement (MSA) . They are compatible with the Gigabit Ethernet and 1000BASE-T standards as specified in IEEE Std 802.3 . The SFP-1000BASE-T physical layer IC (PHY) can be accessed via I2C, allowing access to all PHY settings and features.

The SFP-1000BASE-T uses the SFP's RX_LOS pin for link indication. If pull up SFP's TX_DISABLE pin,  PHY IC be reset.

3. Work Mode

| Work Mode | Support |
| --- | --- |
| Serdes to 1000base-T | Yes (default) |
| SGMII to 10/100/1000base-Tx | Yes (realization by configuring PHY IC via I2C, detail in section 10) |

4.SFP to Host Connector Pin Out

| Pin | Symbol | Name/Description | Ref. |
| --- | --- | --- | --- |
| 1 | VEET | Transmitter Ground (Common with Receiver Ground) | 1 |
| 2 | TFAULT | Transmitter Fault. Not supported. |  |
| 3 | TDIS | Transmitter Disable. Laser output disabled on high or open. | 2 |
| 4 | MOD_DEF(2) | Module Definition 2. Data line for Serial ID. | 3 |
| 5 | MOD_DEF(1) | Module Definition 1. Clock line for Serial ID. | 3 |
| 6 | MOD_DEF(0) | Module Definition 0. Grounded within the module. | 3 |
| 7 | Rate Select | No connection required |  |
| 8 | LOS | Loss of Signal indication. Logic 0 indicates normal operation. |  |
| 9 | VEER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 10 | VEER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 11 | VEER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 12 | RD- | Receiver Inverted DATA out. AC Coupled |  |
| 13 | RD+ | Receiver Non-inverted DATA out. AC Coupled |  |
| 14 | VEER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 15 | VCCR | Receiver Power Supply |  |
| 16 | VCCT | Transmitter Power Supply |  |
| 17 | VEET | Transmitter Ground (Common with Receiver Ground) | 1 |
| 18 | TD+ | Transmitter Non-Inverted DATA in. AC Coupled. |  |
| 19 | TD- | Transmitter Inverted DATA in. AC Coupled. |  |
| 20 | VEET | Transmitter Ground (Common with Receiver Ground) | 1 |

Notes:

1. Circuit ground is connected to chassis ground

2. PHY disabled on TDIS  > 2.0V or open, enabled on TDIS < 0.8V

3. Should be pulled up with 4.7k - 10k Ohms on host board to a voltage between 2.0 V and 3.6 V. MOD_DEF(0) pulls line low to indicate module is plugged in.

Figure 1. Diagram of host board connector block pin numbers and names

5.  +3.3V Volt Electrical Power Interface

The SFP-1000BASE-T  has an input voltage range of 3.3 V +/- 5%. The 4V maximum voltage is not allowed for continuous operation.

| +3.3 Volt Electrical Power Interface |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Supply Current | Is |  | 320 | 375 | mA | 1.2W max power over full range of voltage and temperature. See caution note below |
| Input Voltage | Vcc | 3.13 | 3.3 | 3.47 | V | Referenced to GND |
| Maximum Voltage | Vmax |  |  | 4 | V |  |
| Surge Current | Isurge |  |  | 30 | mA | Hot plug above steady state current. See caution note below |

Caution: Power consumption and surge current are higher than the specified values in the SFP MSA

6. Low-Speed Signals

MOD_DEF(1)  (SCL)  and  MOD_DEF(2)  (SDA),  are  open  drain  CMOS  signals  (see section VII, "Serial Communication Protocol").   Both MOD_DEF(1) and MOD_DEF(2) must be pulled up to host_Vcc

| Low-Speed Signals, Electronic Characteristics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Max | unit | Notes/Conditions |
| SFP Output LOW | VOL | 0 | 0.5 | V | 4.7k to 10k pull-up to host_Vcc, measured at host side of connector |
| SFP Output HIGH | VOH | host_Vcc -0.5 | host_Vcc + 0.3 | V | 4.7k to 10k pull-up to host_Vcc, measured at host side of connector |
| SFP Input LOW | VIL | 0 | 0.8 | V | 4.7k to 10k pull-up to Vcc, measured at SFP side of connector |
| SFP Input HIGH | VIH | 2 | Vcc + 0.3 | V | 4.7k to 10k pull-up to Vcc, measured at SFP side of connector |

7.High-Speed Electrical Interface

All high-speed signals are AC-coupled internally.

| High-Speed Electrical Interface, Transmission Line-SFP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Line Frequency | fL |  | 125 |  | MHz | 5-level encoding, per IEEE 802.3 |
| Tx Output Impedance | Zout,TX |  | 100 |  | Ohm | Differential, for all frequencies between 1MHz and 125MHz |
| Rx Input Impedance | Zin,RX |  | 100 |  | Ohm | Differential, for all frequencies between 1MHz and 125MHz |

| High-Speed Electrical Interface, Host-SFP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Single ended data input swing | Vinsing | 250 |  | 1200 | mV | Single ended |
| Single ended data output swing | Voutsing | 350 |  | 800 | mV | Single ended |
| Rise/Fall Time | Tr,Tf |  | 175 |  | psec | 20%-80% |
| Tx Input Impedance | Zin |  | 50 |  | Ohm | Single ended |
| Rx Output Impedance | Zout |  | 50 |  | Ohm | Single ended |

8.General Specifications

| General |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Data Rate | BR | 10 |  | 1000 | Mb/sec | IEEE 802.3 compatible. See Notes 2 through 3 below |
| Cable Length | L |  |  | 100 | m | Category 5 UTP. BER |

Notes:

1. Clock tolerance is +/- 50 ppm

2. By default, the SFP-1000BASE-T is a full duplex device in preferred master mode

3. Automatic crossover detection is enabled.  External crossover cable is not required

9.Environmental Specifications

| Environmental Specifications |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| Operating Temperature | Top | 0 |  | 70 | °C | Case temperature |
| Storage Temperature | Tsto | -40 |  | 85 | °C | Ambient temperature |

10. Serial Communication Protocol

All WINTOP SFPs support the 2-wire serial communication protocol outlined in the SFP MSA. These SFPs use an MCU, can be accessed with address of A0h.

| Serial Bus Timing, Requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min | Typ | Max | unit | Notes/Conditions |
| I 2C Clock Rate |  | 0 |  | 200,000 | Hz |  |

The SFP-1000BASE-T physical layer IC can also be accessed via the 2-wire serial bus at address ACh. For  details interfacing with the PHY IC, see Marvell data sheet titled "Alaska Ultra 88E1111 Integrated  Gigabit Ethernet Transceiver" (Marvell document number MV-S100649-00).

11. Outline Dimensions (mm)